COMP.CE.400

System Design

Final Report

Group 03

Group member 1 Shelcia Carolyn Samuvel, 153055122, shelcia.samuvel@tuni.fi

Group member 2 name, student number, e-mail address

# Overview of the System

Describe here what kind of system you implemented in the exercise project. What does it do and what are its main parts and features? Focus only on the implementation on the FPGA board, not the design process, like SystemC simulations. You may illustrate the description with an image, if you wish.

In this exercise project, we built a fully integrated hardware–software video encoding and streaming system on a Cyclone V SoC FPGA board. The system captures live video from the onboard camera sensor, sends it through a hardware-accelerated Kvazaar encoder on the FPGA, and then streams the encoded video to a PC (running on a VM) using FFmpeg over UDP. The system also makes use of various FPGA-controlled peripherals such as LEDs, buttons, DMA channels, and custom hardware accelerators.

The main goal of this exercise project was to show how compute-intensive video encoding tasks can be shared between FPGA based hardware accelerators and software running on the HPS. This division of work improves throughput and reduces the CPU load. The exercise project progresses step by step from simple LED control and memory-mapped I/O, to a software only Kvazaar implementation, and finally to a fully hardware-accelerated, real-time HEVC encoding and streaming pipeline.

## Hardware

Describe here the hardware components of the system. Obviously this includes the FPGA board, its add-ons and the PC, but more importantly you should describe what resources (processors, FPGAs, memories, HW accelerators, buses etc. if any) are used on the FPGA board and how they communicate with each other.

The hardware platform for this exercise project is built around the Cyclone V SoC, which comes with:

- A dual-core ARM Cortex-A9 HPS

- FPGA fabric

- HPS–FPGA bridges (including lightweight AXI, full AXI, and AXI-to-channel interfaces)

- Shared DDR memory accessible by the HPS, Linux, and the hardware accelerators

- Camera interface pins

- LEDs and buttons used as GPIO

- Several custom hardware accelerators, including:

  - The Intra Prediction SAD Accelerator (IP_SAD_Accelerator)

  - DMA components for reading and writing YUV image blocks

  - Camera control logic for adjusting brightness, resolution, and capture triggers

All components communicate using memory-mapped AXI interconnects. The HPS interacts with the accelerator registers and buffer spaces through virtual memory–mapped peripheral addresses, while DMA engines handle pixel transfers between DDR memory and the FPGA logic without requiring CPU intervention.

## Software

What software modules are used by the system and what is the purpose of each one? This includes programs and processes that are running when the system is on. How are they mapped on the hardware resources and how do they communicate with each other?

The software stack for this exercise project is made up of several layers and components that work together to control the hardware and manage the video processing pipeline. It includes:

- BlinkerApp, which reads button states and controls LED outputs

- BlinkerHAL, a hardware abstraction layer that simplifies low-level device access

- FPGAHAL, providing core routines for SoC communication

- SystemC models, used in earlier exercises to simulate computation and data flow

- Kvazaar encoder components, which helped with profiling and splitting functionality between hardware and software

At the last stage of the exercise, the stack is organized as follows:

- Linux OS running on the ARM HPS

- Kernel drivers, including:

  - The Kvazaar IP Accelerator driver (ip_acc_driver.ko)

  - The camera driver (camera_driver.ko)

- User-level applications, such as:

  - The hardware-accelerated Kvazaar encoder

  - The cameraUser program for raw YUV capture

  - FFmpeg for video streaming

Communication between the software and hardware happens through:

- Memory-mapped I/O handled via the HAL layers

- Kernel drivers that manage DMA engines and accelerator registers

- Standard I/O pipes connecting Kvazaar with FFmpeg during streaming

Together, these software components coordinate the entire workflow by capturing frames, moving data with DMA, running hardware-accelerated encoding, and streaming the final video; while the FPGA takes care of the heavy pixel-level processing.

# Summary of the Exercises

Write here a short summary of what you did in each of the individual exercises. Emphasize the big picture: what was the purpose of the exercise, what tools you used and what you did with them, and what was the end result. A couple of paragraphs per exercise is sufficient. The purpose of this section is to ensure that you have a good general picture of the exercise project as a whole.

## Exercise 1

In this exercise, we were introduced to the basics of the Cyclone V SoC and how its FPGA fabric works together with the Hard Processor System (HPS). The main goal was to get comfortable with the development tools, Quartus Prime for building and simulating hardware, and Kactus2 for exploring IP components and their connections. By looking at how IP blocks are structured and how they link to the HPS, we started to understand the bigger picture of hardware–software communication on the SoC.

On the practical side, we created and simulated simple Verilog modules, experimenting with clocks, resets, and basic logic operations. Seeing the simulations and linking the modules to the HPS helped us build confidence in designing small hardware blocks and understanding how they interact with software. Overall, the exercise gave us a solid, hands-on introduction to SoC development and prepared us for the more advanced hardware–software integration tasks that came later in the course.

## Exercise 2

In this exercise, we learned how to model hardware-software systems using SystemC. We started by running a C-based encryption/decryption application, then split it into multiple SystemC processes (P1–P4) connected via FIFOs and shared memory. We implemented P3 and P4, synchronized the processes, and verified the simulation using a test bench and GTKWave for signal visualization.

Next, we added timing delays to reflect realistic hardware behavior and optimized performance by adjusting memory delays, bus speeds, and processor assignments, reducing processing time from 670 ns to about 158 ns. The exercise gave hands-on experience with process communication, synchronization, timing analysis, and performance optimization in hardware/software co-design, while also answering conceptual questions about SystemC and memory behavior.

## Exercise 3

In this exercise, we explored untimed SystemC modeling and TLM2.0-based communication. First, we modified the CrypterSimApp to use TLM sockets instead of FIFOs, allowing master and slave processes to transfer data more realistically. Then, we implemented read() and write() functions and TLM communication between the Kvazaar encoder and its intra prediction submodule, verifying that the separated modules worked correctly. Overall, the exercise gave hands-on experience in modular design, software–hardware communication, and preparing software for hardware acceleration.

## Exercise 4

In this exercise, we got hands-on experience with the Cyclone V SoC by creating a simple LED-Blinker application. Buttons on the FPGA board controlled LEDs through software running on the HPS. Using Quartus Prime and Qsys, we set up PIO components, connected them via the AXI bus, and generated addresses for software access. On the software side, the BlinkerHAL layer mapped hardware registers to virtual memory, allowing the application to safely read buttons and control LEDs.

We compiled the application, transferred it to the board, and verified its functionality via the serial terminal. This exercise helped us understand hardware/software co-design, memory-mapped I/O, and the role of HAL layers in separating software from hardware, providing a practical introduction to building integrated FPGA systems.

## Exercise 5

## In this exercise, we benchmarked the Cyclone V SoC to evaluate its memory, I/O, and processing performance. We measured transfer rates using both user-space and kernel-space drivers, and used SignalTap II to accurately capture DMA speeds, which were much higher than application-level measurements. We also calculated the MIPS of the ARM CPU and FPGA, estimated speculative frame rates, and marked key measurement points in the hardware design.

## We then ran the Kvazaar encoder on the ARM CPU to profile performance and memory usage, comparing it to a virtual machine workstation. The results highlighted the performance gap between pure software and hardware-accelerated execution, demonstrating the benefit of offloading compute-intensive tasks to the FPGA. Overall, the exercise gave practical insights into the board’s capabilities and prepared us for leveraging hardware acceleration in later exercises.

## Exercise 6

In this exercise, we explored the performance of the Kvazaar HEVC encoder on the Cyclone V SoC using SystemC simulations. We started with an untimed model, adjusting delay_c and encoder percentages based on profiling data to match the FPS observed on ARM hardware. This gave us a baseline understanding of how the simulation compares to real hardware execution.

Next, we added a timed intra prediction hardware accelerator to see how much it could speed up the encoder. We tracked data transfers, added profiling for different encoder parts, and compared waveforms to understand timing behavior. Finally, we experimented with system parameters like CPU cores, clock speeds, FPGA frequency, and the number of accelerators to explore real-time full HD encoding. The exercise demonstrated how software profiling and hardware acceleration can be combined to optimize compute-intensive tasks.

## Exercise 7

In this exercise, we integrated the Kvazaar IP SAD accelerator and camera control logic on the Cyclone V SoC. We created and connected QSYS components in Kactus2, synthesized the hardware in Quartus Prime, and ensured proper port mapping, addresses, and IRQ connections. Once the hardware was ready, we tested it using kernel drivers and user-space applications. The Kvazaar accelerator sped up video encoding compared to a software-only implementation, while the camera application allowed video capture, highlighting timing issues in continuous mode. This exercise gave hands-on experience in hardware/software co-design, hardware synthesis, driver interaction, and validating accelerated video processing, while also showing practical challenges like debugging connections and handling real hardware constraints.

## Exercise 8

In this exercise, we profiled the hardware-accelerated Kvazaar encoder and set up live video streaming from the Cyclone V SoC board to a PC. We first ran a profiling build on the board, generating a gmon.out file, which showed that kvz_quantize_residual_generic became the main processing hotspot while search functions decreased. For streaming, we tested pre-encoded HEVC files with FFmpeg and then implemented live streaming directly from the hardware-accelerated Kvazaar. Using shell scripts, we controlled encoder parameters like QP, threads, resolution, and camera mode. Performance measurements revealed that resolution had the biggest impact on latency and FPS. Overall the exercise gave practical experience with hardware/software co-design, real-time encoding, and streaming pipelines.

# Suggestions and Feedback

Give us some suggestions on how we could improve the course and the exercise project. Even this section will affect the grade for this exercise so be thoughtful and constructive.

## What Did We Learn

What did you learn on the course as a whole and on the exercise project in particular? Did you learn everything you expected from the course? Were some topics missing?

Throughout the course, we developed a strong and practical understanding of hardware–software co-design on FPGA based SoC platforms, with a particular focus on the Cyclone V SoC. The exercises took us step by step from the fundamentals of FPGA–HPS interaction to building a fully integrated system capable of live HEVC video encoding. One of the most important lessons was learning to evaluate system performance as a whole, not just in terms of computation. We learned to consider memory bandwidth, DMA throughput, communication latency, and how these factors collectively shape real-world performance.

During the exercise project, we had the opportunity to work with advanced design tools such as Quartus Prime, QSYS, and Kactus2. These tools allowed us to design and integrate hardware components, create custom IP cores, connect them using AXI buses, manage interrupts, and map hardware registers so that they could be accessed from software. Working with the hardware-accelerated Kvazaar encoder and the camera interface gave us firsthand experience in offloading heavy computation to hardware, while also highlighting system-level challenges such as timing mismatches, continuous vs. non-continuous capture modes, and delays introduced by DMA transfers.

We also strengthened our skills in performance profiling. By comparing software-only encoding, simulation-based acceleration, and actual hardware-accelerated encoding, we learned how to estimate speed-ups, validate them through measurement, and understand how practical constraints influence final system behavior.

Overall, the course offered an excellent mix of theory and hands-on experience.

## What was Easy/Hard

Which parts of the exercise project did you find the easiest? On the other hand, what was hard and time-consuming?

Easy:

- Working with Kactus2 once the workflow became familiar

- Using provided drivers and run scripts

- Streaming with FFmpeg once network issues were fixed

Hard:

- Qsys component creation, IRQ assignment, and signal mapping

- Debugging port mismatches and kernel driver issues

- Quartus compilation (slow and sensitive to configuration mistakes)

- Profiling and connecting multiple subsystems in Exercise 8

- Corrupted SD card files and VM USB disconnects, also caused delays.

## What Needs Improvement

How would you improve the course and the exercise project?

Some of the later exercises, especially the hardware integration and synthesis tasks in Exercises 7 and 8, were quite time-consuming and could be overwhelming for students with limited FPGA experience. Breaking these tasks into smaller, clearly defined steps with intermediate checkpoints would help us progress more confidently. Additionally, the streaming and profiling exercises sometimes suffered from connectivity issues, particularly with USB Ethernet adapters. Providing alternative streaming methods would ensure that students are not blocked by hardware-specific limitations.